XIAMEN SMARK IMP.& EXP. CO., LTD.

No:(2005)FC001

###### Inspection Reports

Product Name: 密封深沟 Bearing

# Type: 6204-2RS

Applicant: xxx  BEARING FACTORY

## NOTICE

- The inspection reports would be invalid without stamp for inspection report.

- The copy of inspection reports would be invalid without added inspection report or inspection unit stamp.

- The inspection report would be invalid without signatures.

- The inspection report would be invalid if altered.

- If there is any objection to the inspection reports, please claim to inspection unit in 15days.Any claim later than 15 days will not be considered.

- The inspection data is only valid to the samples.

Add: RM#1902, NO.584,DEXIN BUILDING,XIANYUE ROAD,XIAMEN, CHINA

Post Code:361004

Tel:(86)(592)6780643

Fax:(86)(592)6106330

厦门斯玛克进出口有限公司

检   验   报  告   结  论

共3页  第1页

| 报告编号 | NO:FC-2006-001 | 产品名称 | 密封深沟球轴承 |
| --- | --- | --- | --- |
| 型号规格 | 6204-2RS | 检验类别 | 委托检验 |
| 受检单位 |  | 详细地址 |  |
| 商标 |  | 抽样方式 |  |
| 外形尺寸 | 20×47×14 | 抽样地点 |  |
| 公差等级 | P0 | 抽样基数 |  |
| 材质 | GCr15 | 抽样日期 |  |
| 生产日期 |  | 抽样日期 |  |
| 检验日期 | 2006-2-8 | 抽样者 |  |
| 检验地点 | 动态实验室 | 检验依据 | JB/T8571-1997 |
| 检 验 结 论 | 综合判定本批样品为合格品. 检验报告专用章 签发日期 2006 年 1 月 12 日 |  |  |
| 附 注 |  |  |  |

批准:                                  审核:                          编制:

厦门斯玛克进出口有限公司

密封轴承防尘检验报告

共3页  第2页

| 受检单位: 规格型号: 6204-2RS 公差等级: P0 报告编号: FC-2006-001 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 试 验 方 法 | 防尘试验 |  |  | 样本容量 |  |  | 8套 |
| 试 验 条 件 | 试验机型号 |  | TSMO20-45 |  |  |  |  |
|  | 径向负荷(Fr) |  | / N |  |  |  |  |
|  | 轴向负荷(Fa) |  | / N |  |  |  |  |
|  | 试验转速(n) |  | 试验机 |  | 2700 r/min |  |  |
|  |  |  | 灰尘介质箱 |  | 50 r/min |  |  |
|  | 试验时间(h) |  | 6 h |  |  |  |  |
|  | 填灰量(ml) |  | 0.4%V |  |  |  |  |
| 仪 器 仪 表 | 转速 |  | 变频器 |  |  |  |  |
|  | 温度 |  | 温度传感器 |  |  |  |  |
|  | 称重 |  | 精密电子天平 精度≤0.001g |  |  |  |  |
| 试验开始日期 |  | 2006 年1月 10日 | 试验结束日期 |  |  | 2006年 1 月11 日 |  |

厦门斯玛克进出口有限公司

密封深沟球轴承防尘试验数据

共3页  第2页

| 轴承代号 |  |  | 6204-2RS |  |  |  |  |  | 试验编号 |  |  |  |  | FC-2006-001 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 委托试验单位 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 轴承生产厂 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 试验类别 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 试验机型号及编号 |  |  |  |  |  | TSMO20-45 1# |  |  |  |  | 试验转速 |  |  | 2700 r/min 50 r/min |  |
| 灰尘介质 |  |  |  |  |  | 240#的铬刚玉粉 |  |  |  |  | 灰尘填量 |  |  | 0.4V% ml |  |
| 试验时间 |  |  |  |  |  | 6 h |  |  |  |  |  |  |  |  |  |
| 试验开始日期 |  |  |  |  |  | 2006年 1 月 10 日 |  |  |  |  |  |  |  |  |  |
| 试验结束日期 |  |  |  |  |  | 2006年 1 月 11 日 |  |  |  |  |  |  |  |  |  |
| 试验结果 |  | 合格品 |  |  |  |  | 8套 |  |  |  |  |  |  |  |  |
|  |  | 不合格品 |  |  |  |  | 0套 |  |  |  |  |  |  |  |  |
| 序号 | 试验编 号 | 扭矩 (in-zo) |  | 试验时间(h) |  |  | 环境温度 (℃) |  |  | 灰尘侵 入量/cm3 |  |  | 标准灰尘侵入量/cm3 |  | 是否 合格 |
| 1 | F001-1 | 2.0 |  | 6 |  |  | 25.3 |  |  | 1235 |  |  | ≤16000 |  | 合格 |
| 2 | F001-2 | 2.8 |  | 6 |  |  | 26.9 |  |  | 1267 |  |  |  |  | 合格 |
| 3 | F001-3 | 2.4 |  | 6 |  |  | 26.9 |  |  | 1467 |  |  |  |  | 合格 |
| 4 | F001-4 | 1.9 |  | 6 |  |  | 26.7 |  |  | 1560 |  |  |  |  | 合格 |
| 5 | F001-5 | 1.7 |  | 6 |  |  | 25 |  |  | 1356 |  |  |  |  | 合格 |
| 6 | F001-6 | 1.6 |  | 6 |  |  | 29.7 |  |  | 1453 |  |  |  |  | 合格 |
| 7 | F001-7 | 1.8 |  | 6 |  |  | 27 |  |  | 1365 |  |  |  |  | 合格 |
| 8 | F001-8 | 1.5 |  | 6 |  |  | 29 |  |  | 1543 |  |  |  |  | 合格 |
| 以下空白 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

制   表:                               年      月     日             校核:                             年     月    日