Inspection program (example)

DEEP GROOVE BALL BEARING

Ref: IP ABC DGBB- d/m/y

1.	STANDARD EMPLOYED

ISO 2859 and relative standards

- INSPECTION CLASSIFICATION

| KEY ITEMS | MAIN ITEMS | SUB-ITEMS |
| --- | --- | --- |
| HARDNESS | △dmp, Vdp, Vdmp | △Bs, VBs |
| CRACK | △Dmp, VDp, VDmp | △Cs, VCs |
| MARKING | Gr | MAGNET RESIDUE |
|  | Kia, Sia | CHAMFER |
|  | Kea, Sea | APPEARANCE |
|  | Sd, SD | ANTI-RUST PACKING |
|  | ROTATION | SURFACE ROUGHNESS |
|  | SEALING | MARKING QUALITY |
|  | NOISE AND VIBRATION |  |

- ACCEPTABLE LEVEL

Key items	:	AQL 0.1

Main items	:	AQL 1

Sub items	:	AQL 4

- SAMPLING SIZE

ISO 2859 Grade 1

- SAMPLING TYPE

One time sampling type of normal inspection

- SAMPLING BATCH TREATMENT

  - Inspection passed

Whole item consignment batch passed.

  - Inspection failed

Whole item consignment batch returned to suppliers for 100%

Screening before for another inspection.

Example: for a batch size of 10,000 pcs, sample size=80pcs.

Main items: AC(pass number)=2pcs.		Re (failed number)=3pcs.

Sub-items: AC(pass number)=7pcs.		Re (failed number)=8pcs.